Tools Required:

- #1 phillips screwdriver

- #2 phillips screwdriver

- T15 Torx screwdriver

- 7/16” wrench

- Wire strippers

- Heat gun

- Crimping tool (e.g. Klein 3005CR)

- Construct all cables as detailed in the cable construction guide

- Construct mounting plates and tabs

  - Laser cut from ⅛” (3mm) acrylic (example) - can be done in-house or ordered from a manufacturer such as Ponoko (Clear Acrylic, 3mm)

- Assemble battery board

  - Connect cable tabs to the mounting plate using #4 thread cutting screw, #4 nylon washer (between mounting plate and cable tab) and #4 stainless washer (between screw head and cable tab). Do not over tighten screws, these are easy to strip.

  - Connect SunSaver charge controller to mounting plate using ¼” long #6 thread cutting screws and #6 stainless washers. Do not over tighten screws, these are easy to strip.

  - Connect terminal block to mounting plate using ⅜” long #6 Torx thread cutting screws and #6 stainless washers. Do not over tighten screws, these are easy to strip.

- Attach mounting plate to top of battery using VHB tape

- Connect cables as described in the box wiring diagram

  - Charge battery using wall car battery charger

  - Connect solar cable (A) to SunSaver

  - Connect SunSaver to terminal block (C)

  - Connect logger power to terminal block (F)

  - Connect logger data to terminal block (E)

  - Connect sensor cable to terminal block (D)

  - Connect logger data and sensor data together - wire nut

  - Connect battery to SunSaver (B) - Negative first

    - Connect spades to battery posts using ¼”-20 screw and nut with stainless washers on both sides along with lock washer on the nut side - tighten firmly

- Place battery assembly inside of battery box

- Secure cable to tabs using zip ties